# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title: Supply of equipment for Cultural Information Centre "Romansa" within project CB006.1.21.191	p 1 /5

Publication reference: CB006.1.21.191– PP1 – Supply 1

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Cinema projector should be installed in the newly refurbished cinema hall in Makedonska Kamenica where will be projected apart of regular movie shows information about sites of historical and cultural significance and its preservation of tangible and intangible cultural heritage in the region with following specification: Fully integrated digital cinema projector, based on 0.69" DLP Cinema chip or equivalent. It must ensure a first-class movie experience with consistent picture brightness, rich contrast and vibrant colors. Projector must have a screen management stem featuring an intuitive user interface. The projector has to detects potential operational errors and warns users , for instance, a scheduled show is missing assets. It also has to have an easy-to-use, fully modular architecture for simple serviceability and higher uptime. The projector has to come with a third-party media server. DMD 3 x 0.69" DC2K or equivalent Native resolution 2,048 x 1,080 pixels Housing Hermetically sealed DMDs and optical assembly Xenon Lamp 1.6kW, Light output 6.000 lumens Screen size Up to 8m / 26ft wide (screen gain 1 @ 14ftl). Projector should be delivered with prime lense 2.0 - 3.9 ratio, I/O 3x Ethernet 8x GPIO (DB 25) 3D or equivalent and USB interface, Integrated Cinema Media Processor DCI 4K 2D up to 60fps* of equivalent, DCI 4K 3D (24 or 30 fps per eye)*, DCI High Frame Rates 2K 3D up to 120fps (60fps per eye), JPEG 2000 bit-rates up to 625Mbps, Dual-channel color-correction, MPEG-2 (4:2:0 and 4:2:2 up to 60fps), 2x HDMI2.0a (up to 4K 2D 60fps), 2x 3G-SDI inputs, 16x AES/EBU audio channels (2x RJ45), 1.9TB effective storage (RAID-5) with 3x 1TB Hot-swappable 2.5" hard-drives included. Projection system should be delivered with all installation accesories, CineMate App or equivalent, Power cable, pedestal and Dolby Digital Cinema Processor or equivalent, with Dolby Surround 7.1 (D-cinema audio) or equivalent, 5.1 digital PCM (D-cinema audio), Dolby Digital Surround EX™ (bitstream) or equivalent, Dolby Digital (bitstream) or equivalent, and decoder. Processor supports the Software, enabling Eight-Channel Digital Input (4 × AES/EBU) for Digital Cinema Server or equivalent, Includes support for Surround 7.1,Two Digital Pair Inputs (1 × AES/EBU) , Toslink Digital Input, Eight-Channel Analog Input, USB and Ethernet Connector. Supplier is obligated to provide delivery of the goods, instalation and whole system integration with pre installed audio equpment, including all system modifications needed. |  |  |  |
| 2 | Nano projection screen. The surfeace should be 1.7 m +/- 10%; Wide – 3 m +/-10%; Weight – max 20 kg; time to inflate the frame – max 70 seconds; |  |  |  |
| 3 | Cinema projector screen should be installed in the newly refurbished cinema hall in Makedonska Kamenica where will be projected apart of regular movie shows information about sites of historical and cultural significance and its preservation of tangible and intangible cultural heritage in the region with following specification: Cinema Screen should be designed especially for projection to large audiences and should produce bright, sharp images over a broad viewing area. The Screen should be with optical system with two active lens surfaces that offers very good brightness uniformity and contrast levels. Theater screen dimension should be 6.20 (+/-10%) meters by 3.35 (+/- 10%) (20,8 m2) meters projection surface, made of material with following specification: On-Axis Gain 2.4: 1, Ultra-high brightness screen intended for 3D single polarised light systems, 2D digital and 3D digital, Screen Maximum Gain - 2.4 , Material -PVC with aluminium coating or equivalent, Perforation - standard or mini-perf, Weight – max 0.50 kg/m2 Thickness - 0.3mm (+/-10%), completed with eyelts, welded frame ends, elastic bands with hooks and mobile metal construction for cinema scereen 640 x 355 cm (W x H ) minimum dimension, including acoustic baflle and L/C/R/LFE speakers support, fixed masking. |  |  |  |
| 4 | Wide screen smart TV should be installed in the newly refurbished cinema hall in Makedonska Kamenica where will be projected apart of regular movie shows information about sites of historical and cultural significance and its preservation of tangible and intangible cultural heritage in the region with following specification: TYPE-Full HD LED TV or equivalent, minimum Dimensions (W x H x D)-1233 x 766 x 248 Power requirement - AC 100-240 V ~ 50/60 Hz (+/-10%) Technology that allows one remote to control all digital devices in a home theater/entertainment system. High Definition MultiMedia Interface or equivalent. It should allow Stream movies and TV shows, Updating of Facebook, and use Apps right on the screen. |  |  |  |
| 5 | 9 pc. personal computers for preparation of informational materials about sites of historical and cultural significance and its preservation of tangible and intangible cultural heritage in the region and in the public library where the visitors will be able to read about it with following specification: Desktop PC - Processor - Quad Core, 6MB, 2.00GHz w/HD4600 Graphics minimum requirements. Operating System - Windows 7 Professional English or equivalent Memory - minimum 4G (1x4GB) 1600MHz DDR3 Memory Hard Drive - minimum 1TB 2.5inch Serial ATA (7,200 Rpm) Hard Drive Keyboard Mouse Display- min 23.8-inch FHD (1920x1080) Truelife LED-Backlit Touch Display with Wide Viewing Angle (IPS) |  |  |  |
| 6 | 2 pc. laptop PC that will be used for mobile manifestations and regular presentations of sites of historical and cultural significance and its preservation of tangible and intangible cultural heritage in the region with folowing specification: Processor – minimum requirements: Quad Core 2.30GHz, 3.20GHz Turbo, 6MB 45W, HD Graphics 530 Operating System - Windows 7 Professional or equvalent Memory- minimum 8GB, DDR4-2133MHz SDRAM, 1 DIMM Hard Drive – minimum 1 TB 2.5 inch SATA 7200 rpm Hard Drive Graphics Card - AMD FirePro W5170M w/2GB GDDR5 or equivalent Display – min. 17.3" (1920x1080) Wide View Anti-Glare LED-backlit, Wireless - Dual-Band Wireless-AC 8260 Wi-Fi 4.1 Wireless Card (2x2) or equivalent, W/BIuetooth Primary Battery - 6-cell (72Wh) Lithium Ion battery with Ports - 4 USB 3.0 with PowerShare, 1 mDP, 1 HDMI, 1 Headphone and microphone combo jack, 1 SmartCard Reader |  |  |  |