# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Supply of equipment

- Lot 1: Supply of professional public address equipment and indoor 3D projectional equipment	p 1 /…

Publication reference : CB006.1.21.193-BG-01/07.08.2017

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| Professional public address equipment |  |  |  |  |
| 1. Audio mixer - No. of units 1 | •6 microphone/line inputs, plus 3 music source inputs •100 V, call station input with priority •2 zones and 2-channel operation •Voice-activated emergency override •2-tone chime built in, 7 more chimes optional with call station •Outputs for line out, music-only out, and insert point •Separate tone controls for microphones and music sources •Mounting - Stand-alone, 19" rack •Dimensions (H x W x D) 100 x 430 x 270 mm (19" wide, 2U high) |  |  |  |
| 2. 240 WATT AMPLIFIER, 100V - No. of units 1 | • 240 W power amplifier in a compact housing •1 V line level balanced input •70 V, 100 V and 8 ohm outputs •100 V input for slave operation on 100 V speaker line •Temperature-controlled forced front to back ventilation (directly stackable •Dimensions (H x W x D) 100 x 430 x 270 mm (19" wide, 2U high) •Mounting Stand-alone, 19" rack |  |  |  |
| 3. SD/MP3/TUNER BGM SOURCE - No. of units 1 | •MP3 playback from SD card and USB inputs •FM tuner with RDS, presets and digital control •Simultaneous operation of SD/USB-player and FM tuner •Separate outputs for digital source and FM tuner •Dimensions (H x W x D) 44 x 444 x 250 mm (1.73 x 17.48 x 9.84 in) •Mounting Stand-alone, 19" rack (19" wide, 1U high) |  |  |  |
| 4. 30 W PREMIUM SOUND CABINET LOUDSPEAKER - No. of units 8 | •High-fidelity music and speech reproduction •Selectable 8 ohm, 70 V and 100 V inputs •Compact yet robust ABS enclosure •Supplied with adjustable mounting bracket •Complies with international installation and safety regulations •Maximum power: 45 W •Rated power (PHC): 30 W •Weight Approx. 2.4 kg (5.29 lb) •Color White (RAL 9010) or Charcoal (RAL 7021) •Sound pressure level at rated power / 1 W (1 kHz, 1 m): 105/90 dB (SPL) •Effective frequency range (-10 dB): 100 Hz to 18.5 kHz •Opening angle at 1 kHz / 4 kHz (-6 dB): 160° / 81° (hor.)150° / 90° (ver.) |  |  |  |
| 5. CABLES, ADAPTORS, AND CONNECTORS FOR SYSTEM INTEGRATION - No. of units 1 |  |  |  |  |
| Professional wireless microphone system |  |  |  |  |
| 6. WIRELESS MICROPHONE RECEIVER (606-630MHZ) - No. of units 3 | •193 selectable UHF channels •Frequency range 606 to 630 MHz •PLL synthesized technology •Pilot tone & noise squelch •True diversity for stable reception •Table top and 19” rack mountable •HF impedance 50 ohm •Connector XLR 3-pin male, balanced •Output impedance 600 ohm |  |  |  |
| 7. WIRELESS HANDHELD MICROPHONE (606-630MHZ) - No. of units 3 | •193 selectable UHF channels •Frequency range 606 to 630 MHz •PLL synthesized technology •LCD with battery status and frequency indication •Lock function •RF output 10 mW •Approximately 15 hours operation on alkaline batteries •Transducer type Condenser •Antenna Integrated •Polar pattern Unidirectional •Weight 350 g |  |  |  |
| 8. FLOOR STAND FOR WIRELESS HANDHELD MICROPHONE - No. of units 3 | •Height 1600 mm |  |  |  |
| 9. WIRELESS BELT-PACK TRANSMITTER (606-630MHZ) - No. of units 1 | •193 selectable UHF channels •PLL synthesized technology •LCD with battery status and frequency indication •Lock function •RF output 10 mW •Approximately 15 hours operation on alkaline batteries •Connector Mini XLR (tiny QG) •Antenna Flexible •Weight 20 g (0.04 lb) |  |  |  |
| 10. HEAD-WORN MICROPHONE - No. of units 1 | •Optional use with belt-pack transmitters MW1-LTX-Fx •Miniature condenser microphone with omni-directional pick-up pattern •No proximity effect •Polar pattern Omni-directional •Transducer Condenser •Weight 32 g (0.07 lb) •Cable length 1.45 m (4.76 ft) |  |  |  |
| 11. CABLES, ADAPTORS, AND CONNECTORS FOR SYSTEM INTEGRATION - No. of units 1 |  |  |  |  |
| Professional digital discussion microphone system |  |  |  |  |
| 12. CONTROL UNIT WITH RECORDING AND DAFS - No. of units 1 | •Digital •Plug-and-play functionality for connection of up to 80 Discussion Devices • Web browser interface control for advanced configuration and control •Built-in MP3 recorder with internal memory and USB recording •Built-in Digital Acoustic Feedback Suppression (DAFS) for ensuring superior speech intelligibility •XLR nominal microphone input -56 dBV •Mounting Tabletop or 19 “ rack •Weight Approx. 3.4 kg (7.5 lb) |  |  |  |
| 13. DISCUSSION DEVICE WITH FIXED MICROPHONE - No. of units 4 | •Digital •Possible-to-speak indicator •Plug and play functionality for installation and reconfiguration •Configurable as a participant device or chairperson’s device •Immunity to mobile phone interference Microphone polar pattern Cardioid Weight Approx. 1 kg (2.2 lb) Length of microphone from mounting surface: 480 mm (18.9 in) |  |  |  |
| 14. EXTENSION CABLE 5M - No. of units 4 | •Digital •6-pin |  |  |  |
| 15. EXTENSION CABLE 20M - No. of units 1 | •Digital •6-pin |  |  |  |
| 16. ADAPTORS, AND CONNECTORS FOR SYSTEM INTEGRATION - No. of units 1 |  |  |  |  |
| Professional digital simultaneous interpretation system |  |  |  |  |
| 17. CENTRAL CONTROL UNIT - No. of units 1 | •Digital •Control maximum 245 wired contribution units, plus 245 wireless delegate units •Voice activated microphone mode for wired and wireless systems •Ethernet connection for configuration and control computers •Optical network supply 40 Vdc, max 65 W •Mounting: Tabletop or mounted in a 19” rack 2U |  |  |  |
| 18. DIGITAL 4 CHANNEL TRANSMITTER WITH OPTICAL LINK - No. of units 1 | •Digital •Distribution of minimum 4 audio channels •Usable with digital or analogue systems •Flexible configuration of channels and channel quality modes for efficient distribution •Optical network supply 40 Vdc •Mounting: Tabletop or mounted in a 19” rack 2U |  |  |  |
| 19. INFRARED RADIATOR - No. of units 2 | •Digital •Covers at least 1000 m2 up to 1300 m2 (one carrier, 4 standard quality channels) •Automatic gain control ensures the IREDs (infrared emitting diodes) function •Power output selection •Safety eye •Total optical peak intensity 12 W/sr •Dimensions (H x W x D) without bracket 200 x 500 x 175 mm(7.9 x 19.7 x 6.9 in) •Weight: without bracket 6.8 kg (15 lb) |  |  |  |
| 20. MOUNTING BRACKET FOR INFRARED RADIATOR - No. of units 2 | •Dimensions (H x W x D) 200 x 280 x 160 mm(7.9 x 11.0 x 6.3 in) •Weight 1.8 kg (4.0 lb) •Color Quartz grey |  |  |  |
| 21. NETWORK CABLE ( OPTICAL FIBER), 0,5M - No. of units 1 |  |  |  |  |
| 22. POCKET RECEIVER 4 CHANNEL - No. of units 40 | •Digital 4-channels •Recharging electronics integrated in the chip, •2-digit LCD display with battery and reception status indication •Number of available channels is always the same as the number of channels in use by the system, eliminating the need to scroll through unused channels •IR irradiance level 4 mW/m2 per carrier •Headphone output impedance 32 ohm to 2 kohm •Dimensions (H x W x D) 155 x 45 x 30 mm(6.1 x 1.8 x 1.2 in) •Weight excluding battery 75 g (0.16 lb) •Color Charcoal with silver |  |  |  |
| 23. NIMH BATTERY PACK (SET OF 10 PCS) - No. of units 4 | •Temperature sensor •Voltage 2.4 V •Capacity 1100 mAh •Dimensions (H x W x D) 14 x 28 x 50 mm(0.6 x 1.1 x 1.9 in) •Weight 50 g (0.11 lb) |  |  |  |
| 24. CHARGING CASE FOR 56 RECEIVERS - No. of units 1 | •Can accommodate 56 pocket receivers •Universal mains power facility •Rapid recharging: within 1 hour and 45 minutes •Power consumption 300 W (56 receivers charging) •Power consumption (standby) 17 W (no receivers in the charging unit) |  |  |  |
| 25. ADAPTORS, AND CONNECTORS FOR SYSTEM INTEGRATION - No. of units 1 |  |  |  |  |