# VOLUME 3

# TECHNICAL SPECIFICATIONS

GENERAL REQUIREMENTS

The Technical Specification is an integral part of the Tender Documentation /TD/ together with the provisions of the Contract, the detailed design drawings, building permissions and the other contract documents. The Specification specifies and further develops the requirements for the implementation of the construction works under the Contract.

Implementation of construction and assembly work must be consistent with all relevant execution of specific types of works legal and regulatory framework, technical rules and regulations and applicable standards following sequence and technology of the performance of different types of works in different parts of the site.

- STANDARDS

As a minimum the Macedonian standards and codes shall always be satisfied. Other internationally acknowledged standards and codes may be used only if:

- They are more or at least equally stringent compared to the respective Macedonian standards and codes or

- European technical approvals (with or without guidance).

The materials which are used must correspond to the requirements of the standardized documents asset in Macedonian standards and codes.

If the Contractor should wish to supply material or execute work to an alternative national standard or international specification he shall give full details of his proposal in writing to the Supervisor.

- CONTROL OF WORKS

The Contracting Authority will provide a consultant who will supervise construction with investment functions, according to Macedonian legislation and other legal acts in construction works. The Contracting Authority and / or Supervisor may at any time inspect the work, control technology performance and issue instructions to remove the defects, according to the specified technology and method of implementation. If found serious defects, errors and low quality performance, the Contracting Authority shall notify the Contractor that breached the contract and should stop to work. The Contractor shall always provide access to the construction site of the authorized representatives of the Contracting Authority and the Supervisor.

2.1  Contractor's equipment

The Contractor shall furnish equipment which will be efficient and appropriate to secure a satisfactory quality of work and a rate of progress which will insure the completion of the Works within the time stipulated in the Tender. If at any time such equipment appears to be inefficient, inappropriate or insufficient for securing the quality of work required or for the rate of progress, the Supervisor may be entitled to order the Contractor to increase the efficiency, change the character or hire additional equipment, and the Contractor shall conform to such order.

2.2  Protection of existing structures and utilities

The Contractor shall assume full responsibility for the protection of all buildings, structures and roads existing in the area of the construction site, public or private, whether or not they are shown on the drawings.

The Contractor has to pay special attention to avoid any damages on any protected areas.

Any damage resulting from the Contractor's operations shall be repaired at his expense.

2.3	Safety and security on site

Safety and security arrangement should be performed in accordance with Macedonian Construction Law

2.4	Handling and storage of materials and plants

All materials and plants to be incorporated in the work shall be handled and stored in a manner, which prevents injury of any kind whatsoever. Any materials or plants which, in the opinion of the Supervisor, have become too damaged to be fit for the use intended or specified shall be promptly removed from the site, and the Contractor shall receive no compensation for the damaged material or its removal.

2.5	Approval of sources, materials and plants

Materials incorporated must originate in a Member State of the European Union or a country covered by the Interreg - IPA CBC Programme 2014 – 2020. However, the goods to be purchased may originate from any country, whenever the total price of the estimated quantity of those goods, as reflected in a separate item of the Breakdown of the Lump-sum Price (Volume 4.2.3) is below 100.000 €. Material that originates from sources, which are not approved by the Contracting Authority and/or the Supervisor, cannot be used for Works.

Approval of a source does not mean that all material in the source is approved. The Contractor has to ascertain by continuous control check measurements that only material which complies with the requirements specified in the various clauses of these specifications will be used for the Works.

2.7 Clean-up work

The Contractor shall clear way and remove from the site any wreckage, rubbish and temporary works, which are no longer required.

2.8	Responsibility of the Contractor

Approvals from the Supervisor do not relieve the Contractor from his obligations or responsibilities under the Contract.

3.	ADMINISTRATIVE SPEIFICATIONS.

3.1 Progress meetings

The Contractor shall agree with the Supervisor and the Contracting Authority for dates for regular progress meetings. These meetings shall normally be held monthly, no later than 10 working days after the completion of each month.

3.2 Quality assurance

The Contractor shall institute a quality assurance system to demonstrate compliance with the requirements of this contract. The system shall be in accordance with the details stated in the Contract. The Supervisor shall be entitled to audit any aspects of the system.

4.	IMPLEMENTATION OF THE WORKS

4.1	Materials

The Contractor shall use only materials that conform to the technical requirements set forth in the clauses of this Technical Specification.

All Materials and Plant supplied to perform the Permanent Works under the contract shall be new products. Second-hand Materials and Plant will not be accepted.

The Tender drawings constitute the drawings issued for construction/ installation/ execution.

4.2	Testing

Reliable shall be only the type and amount of tests performed in conformity with the prescription of this Technical specification, except when this right is granted to the Contracting Authority.

The Contracting Authority may require additional tests when the results obtained are uncertain.

Beside the tests specified in this Specification, the Contracting Authority may require additional tests to establish possible hidden omissions and effects. Costs for these tests shall be entirely at the Contractor’s expense if such defects are confirmed.

4.3	Inspection and measurement of works

The Contracting Authority may at any time inspect the quality and measure the amount of works performed. If this cannot be done with the Contractor’s assistance, a deadline shall be fixed for hiring external specialists. In this case, the expenses shall be paid by the Contractor.

5.	PREPARATORY WORKS

Before starting the works the Contractor has to perform some preparatory works at the site.

5.1	Boards and signs

The Contractor shall mount and maintain in good condition a board with the name of the project and the co-financing institutions written in a way /text and font size/ conforming to the requirements of the Contract and in compliance with EU visibility guidelines.

5.2 Setting out the site

The Contractor shall in co-operation with the Contracting Authority set out the total site to be used for construction.

5.3 Temporary facilities

The Contractor shall effects all expenditure for establishing, operation and removal of temporary facilities if such are needed for the good performance of the Contract. All needs for establishing such facility shall be duly justified.

5.4 General supply facilities

- Sanitary Arrangements

The Contractor shall provide for and maintain temporary sanitary facilities on the site for the use of all persons connected with the Works. The Contractor shall keep the facilities in a clean and sanitary condition, and shall post notices and take such precautions as may be necessary to keep the site clean.

- Water supply

The Contractor shall provide for and maintain an adequate supply of potable water for his use. The water supply shall be used for construction purposes and for consumption in the temporary facilities.

- Power supply

All electrical power required by the Contractor shall be provided by him at his own expense. The Contractor prior to taking-over of the Works shall remove all temporary installations if it is not agreed upon that the Contracting Authority takes over the installations.

6. ADDITIONAL SPECIFICATIONS

SUBJECT: Remodeling kindergarten building “Detska radost” – branch 2, Strumica Municipality

In process of implementation of planned works activities the contractor must strictly follow attached basic project for reconstruction of JOUDG “DETSKA RADOST”, Strumica - branch No.2 contains mutually aligned individual projects with a separate intention giving a technical solution of the existing facility:

- Architectural project

- Project for plumbing and sewage

- Electro-technical project and

- Thermo-technical project.

Architectural project

The branch No.2 of JOUDG “DETSKA RADOST” - Strumica is a facility made of hard construction on flat terrain which uses materials such as brick, concrete and wood. The basic premises on the ground floor consist of playrooms for toddlers, with accompanying functions for a smooth operation of the primary activity - childcare of preschool age.

The construction is skeletal and consists of reinforced concrete posts and beams. The mezzanine construction in a part of the facility is concrete, cross sectional reinforcement. The roof construction is a wooden grid, and the roof covering is made of ribbed corrugated cement sheets, forged on planked formwork.

A complete outer reconstruction of the kindergarten - Strumica, branch No.2, will consist of:

- Removing the corrugated cement sheets from the roof construction and covering the roof of the facility with plastic sheets

- Construction of a thermal insulation façade (D=10cm)

- Setting PVC windows with 6 chambers, glazed with low energy KA glass.

Project for plumbing and sewage

Based upon the architectural solution, there is a solution made for plumbing and sewage installation for the branch No.2 of JOUDG “DETSKA RADOST” - Strumica.

The plumbing installation consists of an internal and outer installation. For the internal installation polypropylene pipes with appropriate PP fittings and PP valves under every sanitary object will be used; for the outer plumbing lines HDPE polyethylene pipes of 10 bar with fitting elements will be used.

For the inner and outer fecal and atmospheric sewage PVC sewage pipes with appropriate fitting elements will be utilized. For the fecal and the atmospheric sewage will be combined revision shafts towards the applied detail with a medium type lid. The sewage will join the outer sewage system.

Electro-technical project

A third component of the basic project for reconstruction of JOUDG “DETSKA RADOST”, Strumica - branch No.2 is the preparation of the electro-technical project.

Within this project are the following activities:

- Assembly of classical lightning installation (Faraday’s cage) of the branch No.2

- Replacement of the energy inefficient light bulbs with energy saving lamps based on LED technology

- Setting of new 13 candelabras in the yard of the kindergarten with energy saving compact fluorescent bulbs (22 W)

- Installment of an independent photovoltaic system of 6,7 Kw in the kindergarten in Strumica - branch No.2. This system will work independently in relation to the main power supply and will serve to power the internal lightning in the kindergarten and the existing and newly designed candelabras for the yard lightning.

Thermo-technical project

The last component of the basic project for reconstruction of JOUDG ”DETSKA RADOST”, Strumica - branch No.2 consists of a project for central hot water heating, through a thermo-technical project.

According to the project assignment, a system for solar heating of sanitary hot water needs to be designed. The system will be of a closed type with an obligatory circulation of the heated medium (antifreeze) with a built-in pump on the returning pipe of the system.

This system consists of: 4 solar collectors, automatic valve for ventilation, hot water reservoir (boiler 500L), circulation pump, insulation pipes, valves, expansion dish, differential thermostat, thermo manometer and coupling elements.

The hot water will be used for restroom needs, kitchens and other needs, and its quantity is sufficient according to the purpose for which it is used.

SUBJECT: Remodeling kindergarten building “Detska radost” – branches 3 and 4, Strumica Municipality

In process of implementation of planned works activities the contractor must strictly follow attached basic project for reconstruction of JOUDG “DETSKA RADOST”, Strumica – branches 3 and c 4.

Project for solar heating of sanitary water, plumbing and sewage

For the facility of JOUDG “Detska Radost” - Strumica, branch 3 (near the city hospital) and branch 4 (near the Red Cross organization) in the Municipality of Strumica a project was prepared for plumbing and sewage.

The plumbing installation consists of an internal and outer installation. For the internal installation polypropylene pipes with appropriate PP fittings and PP valves under every sanitary object will be used. The pipes are set on a wall 30 cm from the floor and clamped on the wall with pipe hangers. In order to connect the sanitary objects, adapter knee type joints are used. The heights of assembling the plumbing fixtures for the sanitary objects from the floor are:

- For sink - 50 cm

For the outer plumbing lines are planned HDPE polyethylene pipes of 10 bar with fitting elements.

For the inner and outer fecal and atmospheric sewage are planned PVC sewage pipes with appropriate fitting elements. The pipe’s diameters are chosen according to the existing technical regulations. For the outer lines, a fall of 1% of the pipes is predicted and for the inner lines 2%. The inner pipes are attached with clamps. The verticals go near the edges of the room through which they pass so they can be coated. The flows of the sanitary objects are 50-60 cm from the floor. For the fecal and the atmospheric sewage will be combined revision shafts towards the applied detail with a medium type lid. The sewage will join the outer sewage system.

With the hydraulic calculations we check if the planned plumbing pipeline will have enough pressure on the leaking places. The pipeline is designed in a way that the fall of the pressure, which is created inside, will be lower than the pressure VVM and to remain for a normal working of the armature. The pressure which will remain should be bigger than 0.5 bar (5 meters VS).

The amount of the water depends of the number of users, type and number of leaking places, usage and purpose, weather season, culture, traditions and habits. The amount of water leaking through a certain section is calculated by the unit of load JO. With this unit the calculations in the installation become simple. One JO represents the amount of water which leaks on a leaking place through the pipe with F10 mm on maximum jet stream and pressure of leakage of 0.5 bar.

The functional content of the facilities branch 3 and branch 4 consist of: entry space, hallways, play rooms, bathrooms, nurseries, kitchens, office space for the staff and many others. The plumbing and sewage installation is made everywhere according to the rules and regulations.

In the facility BRANCH 3 are built (planned) these plumbing fixtures:

20 sinks, outer fountain, 22 tanks in the bathrooms, three dishwashers, 8 showers.

In the facility BRANCH 4 are built (planned) these plumbing fixtures:

20 sinks, outer fountain, 20 tanks in the bathrooms, 3 dishwashers, 8 showers.

According to the project assignment, a system for solar heating of sanitary hot water needs to be designed. The system will be of a closed type with an obligatory circulation of the heated medium (antifreeze) with a built-in pump on the returning pipe of the system.

In the facility BRANCH 3, the system consists of: 6 solar collectors, automatic valve for ventilation, hot water reservoir (boiler 800 L), circulation pump, insulation pipes, valves, expansion dish, differential thermostat, thermo manometer and coupling elements.

In the facility BRANCH 4, the system consists of: 4 solar collectors, automatic valve for ventilation, hot water reservoir (boiler 500 L), circulation pump, insulation pipes, valves, expansion dish, differential thermostat, thermo manometer and coupling elements.

The hot water will be used for the needs of the bathrooms, kitchens and sinks that are placed on the ground floor of the kindergarten. The amount of the hot water is sufficient for the purposes for which it will be used. A design of the system and the way of connecting the plumbing installation is given in the graphical attachments.

Electro-technical project

In the facilities of JOUDG “Detska Radost” BRANCH 3 and BRANCH 4 in Strumica, an electro-technical project has been prepared. In terms of the project assignment, it is planned to replace all the fluorescent pipe bulbs and the wolfram light bulbs with LED light bulbs. During the replacement of the light bulbs, it is necessary to be done an appropriate adjustment of the existing installation in the armatures with appropriate parts for installation of the LED light bulbs.

According to the project assignment, in both kindergartens a replacement of all old light bulbs with new saving LED light bulbs is needed.

For the facilities (BRANCH 3 and BRANCH 4) these types of LED light bulbs are planned:

- Light bulb Е27 type with the following characteristics:

- Power 10 W

- Power of luminous flux 1080 lm

- Input voltage AC 85-265V

- Color temperature 5000 К

- Light bulb or pipe LED with L=1,2 m with the following characteristics:

- Power 18 W

- Power of luminous flux 1600 Lm

- Input voltage AC 85-265V

- Color temperature 6000 К

- Light bulb or pipe LED with L=0,6 m with the following characteristics:

- Power 10 W

- Power of luminous flux 800 Lm

- Input voltage AC 85-265V

Color temperature 6000 К.

Measures of security and technical protection

In addition there will be included the necessary security measures and technical protection.

- To respect and enforce all laws and regulations in the operation of this type of work, and the regulations  written by the employer;

- To noticing all the necessary requirements and conditions that are part of this project, in terms of standards, regulations and legal provisions for performing this type of work;

- To take care for personal protection of the employees during the working actions  in accordance prescribed legal provisions and regulations;

- During transportation, all participants are obliged to use personal protective equipment.

In process of implementation of all planned works activities the contractor must strictly follow Construction Law of the Republic of Macedonia, the Law for Energy, the Regulator for energy performance of buildings, the Law for construction and other applicable regulations and technical norms of the Republic of Macedonia.