# VOLUME 3

# TECHNICAL SPECIFICATIONS

GENERAL REQUIREMENTS

The Technical Specification is an integral part of the Tender Documentation /TD/ together with the provisions of the Contract, the detailed design drawings, building permissions and the other contract documents. The Specification specifies and further develops the requirements for the implementation of the construction works under the Contract.

Implementation of construction work must be consistent with all relevant execution of specific types of works legal and regulatory framework, technical rules and regulations and applicable standards following sequence and technology of the performance of different types of works in different parts of the site.

- STANDARDS

As a minimum the Macedonian standards and codes shall always be satisfied. Other internationally acknowledged standards and codes may be used only if:

- They are more or at least equally stringent compared to the respective Macedonian standards and codes or

- European technical approvals (with or without guidance).

The materials which are used must correspond to the requirements of the standardized documents asset in Macedonian standards and codes.

If the Contractor should wish to supply material or execute work to an alternative national standard or international specification he shall give full details of his proposal in writing to the Supervisor.

- CONTROL OF WORKS

The Contracting Authority will provide a consultant who will supervise construction with investment functions, according to Macedonian legislation and other legal acts in construction works. The Contracting Authority and / or Supervisor may at any time inspect the work, control technology performance and issue instructions to remove the defects, according to the specified technology and method of implementation. If found serious defects, errors and low quality performance, the Contracting Authority shall notify the Contractor that breached the contract and should stop to work. The Contractor shall always provide access to the construction site of the authorized representatives of the Contracting Authority and the Supervisor.

2.1  Contractor's equipment

The Contractor shall furnish equipment which will be efficient and appropriate to secure a satisfactory quality of work and a rate of progress which will insure the completion of the Works within the time stipulated in the Tender. If at any time such equipment appears to be inefficient, inappropriate or insufficient for securing the quality of work required or for the rate of progress, the Supervisor may be entitled to order the Contractor to increase the efficiency, change the character or hire additional equipment, and the Contractor shall conform to such order.

2.2  Protection of existing structures and utilities

The Contractor shall assume full responsibility for the protection of all buildings, structures and roads existing in the area of the construction site, public or private, whether or not they are shown on the drawings.

The Contractor has to pay special attention to avoid any damages on any protected areas.

Any damage resulting from the Contractor's operations shall be repaired at his expense.

2.3	Safety and security on site

Safety and security arrangement should be performed in accordance with Macedonian Construction Law

2.4	Handling and storage of materials and plants

All materials and plants to be incorporated in the work shall be handled and stored in a manner, which prevents injury of any kind whatsoever. Any materials or plants which, in the opinion of the Supervisor, have become too damaged to be fit for the use intended or specified shall be promptly removed from the site, and the Contractor shall receive no compensation for the damaged material or its removal.

2.5	Approval of sources, materials and plants

Materials incorporated must originate in a Member State of the European Union or a country covered by the Interreg - IPA CBC Programme 2014 – 2020. However, the goods to be purchased may originate from any country, whenever the total price of the estimated quantity of those goods, as reflected in a separate item of the Breakdown of the Lump-sum Price (Volume 4.2.3) is below 100.000 €. Material that originates from sources, which are not approved by the Contracting Authority and the Supervisor, cannot be used for Works.

Approval of a source does not mean that all material in the source is approved. The Contractor has to ascertain by continuous control check measurements that only material which complies with the requirements specified in the various clauses of these specifications will be used for the Works.

2.7 Clean-up work

The Contractor shall clear way and remove from the site any wreckage, rubbish and temporary works, which are no longer required.

2.8	Responsibility of the Contractor

Approvals from the Supervisor do not relieve the Contractor from his obligations or responsibilities under the Contract.

3.	ADMINISTRATIVE SPEIFICATIONS.

3.1 Progress meetings

The Contractor shall agree with the Supervisor and the Contracting Authority for dates for regular progress meetings. These meetings shall normally be held monthly, no later than 10 working days after the completion of each month.

3.2 Quality assurance

The Contractor shall institute a quality assurance system to demonstrate compliance with the requirements of this contract. The system shall be in accordance with the details stated in the Contract. The Supervisor shall be entitled to audit any aspects of the system.

4.	IMPLEMENTATION OF THE WORKS

4.1	Materials

The Contractor shall use only materials that conform to the technical requirements set forth in the clauses of this Technical Specification.

All Materials and Plant supplied to perform the Permanent Works under the contract shall be new products. Second-hand Materials and Plant will not be accepted.

The Tender drawings constitute the drawings issued for construction/ installation/ execution.

4.2	Testing

Reliable shall be only the type and amount of tests performed in conformity with the prescription of this Technical specification, except when this right is granted to the Contracting Authority.

The Contracting Authority may require additional tests when the results obtained are uncertain.

Beside the tests specified in this Specification, the Contracting Authority may require additional tests to establish possible hidden omissions and effects. Costs for these tests shall be entirely at the Contractor’s expense if such defects are confirmed.

4.3	Inspection and measurement of works

The Contracting Authority may at any time inspect the quality and measure the amount of works performed. If this cannot be done with the Contractor’s assistance, a deadline shall be fixed for hiring external specialists. In this case, the expenses shall be paid by the Contractor.

5.	PREPARATORY WORKS

Before starting the works the Contractor has to perform some preparatory works at the site.

5.1	Boards and signs

The Contractor shall mount and maintain in good condition a board (printing and installation of 2 permanent and 2 temporary information signs - at the beginning and at the end of the construction section for river Kozjacka) with the name of the project and the co-financing institutions written in a way /text and font size/ conforming to the requirements of the Contract and in compliance with EU visibility guidelines.

5.2 Setting out the site

The Contractor shall in co-operation with the Contracting Authority set out the total site to be used for construction.

5.3 Temporary facilities

The Contractor shall effects all expenditure for establishing, operation and removal of temporary facilities if such are needed for the good performance of the Contract. All needs for establishing such facility shall be duly justified.

5.4 General supply facilities

- Sanitary Arrangements

The Contractor shall provide for and maintain temporary sanitary facilities on the site for the use of all persons connected with the Works. The Contractor shall keep the facilities in a clean and sanitary condition, and shall post notices and take such precautions as may be necessary to keep the site clean.

- Water supply

The Contractor shall provide for and maintain an adequate supply of potable water for his use. The water supply shall be used for construction purposes and for consumption in the temporary facilities.

- Power supply

All electrical power required by the Contractor shall be provided by him at his own expense. The Contractor prior to taking-over of the Works shall remove all temporary installations if it is not agreed upon that the Contracting Authority takes over the installations.

6. ADDITIONAL SPECIFICATIONS

All provisions and clauses from the Macedonian Construction Law and other codes that are valid obligate the Contractor.

SPECIFIC REQUIREMENTS

River Kozjacka is a left tributary of the river Bregalnica basin of northwestern exposure. Springs are formed from the western slopes of Mount Plackovica the main source of altitude 1240 meters above sea level, the river Vrteska and sources of river Kodzha Dere 1100 meters altitude.

In the composition of these two rivers form Kozjacka river to the confluence of the river Bregalnica has northwestern direction of flow. In the upper river Kozjacka has developed hydrographic network and wide basin with greater forest and mild slopes, while the middle reaches of the river have seen quite pronounced narrowing of the basin, a rare forest and steep slopes to the monastery Kozjak.

Of this section until the confluence of the river Bregalnica river takes lowland character and shaped progress only in terms of larger waters. In periods of less water, especially in the summer months lower flow remains without water, the main reasons deposit of water and extraction of the water for irrigation. Elevation Kozjacka River Basin extends from the highest elevation of 1569 meters above sea level on top of Dzalija to the lowest point at the confluence of the river Bregalnica at Karbinci  village in elevation of 280 meters above sea level.

The presumed height distribution in the catchment areas of the river Kozjacka, notes that the largest share in the total catchment area have areas with an altitude of 800-1000 meters above sea level with representation of 23 km², or about 50% of the total area of ​​the watershed to the considered profile.

Kozjacka natural bed of the river in recent years does not provide sufficient bandwidth to safely miss the big waters flooding after torrential rains on the slopes of Mount Plackovica the catchment area of ​​the river Kozjacka.

By morphological view existing bed undergoing deformation of the part over the bridge where the existing dykes (existing bed) is deformed and insufficient purification of sludge and obstacles that arise under the bridge it comes to reducing the flow so it comes to spill over the bridge section. The section of the bridge towards the river Bregalnica through existing cascading skirts and leveled their good placement does not arise filling the bottom. Only part of the corner about 200 m from the bridge on the right side of the river has come to undermine the embankment with bigger sizes there is an outbreak of helical flow in turn causing destruction of concave mound. All that can be seen from geodesic recording is performed after the flood ie made cross-sections of an existing bed. It can be seen from the accompanying photographs of the current state of the river

According to hydrological analysis undertaken Kozjacka River Centennial leading low probability of occurrence amount Q100 = 48.0 m3/sec, therefore, together with the quantities of rainwater Qatm = 202 m3/sec will be taken as authoritative quantity dimensioning trough regulation the riverbed. This amount of Qmax = 250 m³/sec are shown in this year's floods that hit the municipality of Karbinci and come to fulfillment and spill over the bridge section. So the further dimensioning taken this amount of maximum results.

The level of the existing riverbed is formed with cascades that are pre-formed, due to the small decline in route . .New Projected leveling route ,vertical alignment of the route will be the same slope J = 0.60% as the remaining existing cascading thresholds.
When performance is required in the operation of the route of construction machinery to continuously record the route of the vertical alignment of the riverbed and the embankments of the river bed. On the subject of section elected a structural profile in the shape of a trapezoid, and sizing is done Qmax = 250m3/sec.

The hydraulic calculations for throughput capability of the regulated river bed made and adopted a kind of typical cross section that will fit in the surroundings.

Key elements of the profile in the form of a trapeze are:

Width bottom: b = 18 m

Maximum height of filling: h = 1.70 m

Overall height of the river bed h = 1.50 m

Steepness of the slopes of the basin: 1: 1

As required height can be obtained by dike, because there is not enough space as it should and walkway where flood of great waters they will be prevented from overfill protection with reinforced concrete fence with a height of 1.00 m above ground and reinforcement of its adoption constructive because the pressures are balanced, and it does not need a static calculation. The same applies to the stairs to join the promenade.

Reinforcing the basin is provided by coating the slope of crushed stone and concrete MB30. Below it has a coating of sand and rubble stone with a thickness of 10 cm. The minimum dimension stone cladding needs to be 15 centimeters. Because these are existing thresholds with waterfall and they are already in operation more than 60 years and the appearance of them will damage and undermine them, and no occurrence of rejection jump over existing waterfall, with no need for calculation and dimensioning them.

Transmission of sediment

The occurrence of the traction roller and sediment in natural water flows and transfer in the bed of the watercourse is the result of eroziskite processes primarily of fluvial erosion. General intensity of erosion can determine the transfer of sediment through one or more hydrometric profiles of the natural water flow.

Situational solution

During the preparation of horizontal decision took into consideration the same thing to adjust the existing cascading thresholds, which enabled rapid flow of river water.

The input and output are provided intake thresholds, intended to prevent the occurrence of deformation or damage of the regulated river basin of the river Kozjachka.

.
The dimensioning of these cascade thresholds used the same procedure as for the dimensioning of the cross section of the tray table for the same maximum amount of water.

The situational horizontally solution is shown in graphical attachments in a 1: 1000.  The positioning of the shaft is a list of numeric data for the left and right edges of each profile are marked coordinates X and Y.

Longitudinal solution

The current state of the longitudinal profile is determined by the position of the level of the riverbed of the river Kozjachka and the existing cascading thresholds.

The longitudinal slope varies with constant max gradient of 0.6%.

In graphic attachments has a longitudinal profile in M 1: 1000/100 and is shown with a section of km  0+000.00  to  km 0+862.36.

Transverse profiles

For the subject section are designed transverse profiles with all the necessary information for further use in the performance of the object  in M 1: 200 of the level of this same project given in the graphic part of the project.

Facilities (Objects)

Along the section it register existing cascading skirts and two bridges. Bridging the two bridges is at km 0 + 415.00 of a steel pedestrian bridge km 0 + 51.69 and a regional bridge. Following along the riverbed was obtained information that under the riverbed near the steel pedestrian bridge passes existing water supply and during the performance need to be careful to not make some damages to it.

- Bill of quantities and calculation of the works

- Bill of quantities is designed to cover all positions defined by the technological process for the construction of such facilities. All amounts of work  are calculated  within the scope.

- According to the bill for the amount of work for each position according design  aware of the price of each position, made a bill of quantities and calculation of the works.

Display technology for manufacturing and quality of materials to be applied in the preparation of the object

The technological process starts with clearing, marking and denote the route. When the field is prepared it starts with operational matters according to  the operational plan that is prepared by the contractor and which consists of earthworks - wide excavation with rolled off on the deployment of local planning and forming the profile of the excavation combined clawed hand where needed.

During the excavation is necessary to work continually per parts and if  necessary to divert water for smooth operation. In addition to keeping an eye on weather conditions or to monitor weather forecasts to avoid damage to the already prepared positions. Parallel depending on the operating plan can start making foundations and slopes where they should not be doing time rupture between the foundation and slope. To use local materials by local community which is examined in this kind of work.

Foundations and slopes are provided to bring the stone combined with cement mortar with a well sealed fugues and dimensions given in graphic contributions made to the project. To align the existing slope with the newly been talking of necessary construction work such as demolition or upgrades. In constructing the slopes with stone and cement mortar for building be used rubble stone and river material as outlined in graphic detail.

As a second stage in parallel can be performed reinforced concrete security fence consisting of incorporating a bed of concrete carcass MB15 d = 15 cm on which to set up the necessary fittings to the wall. After the completion of reinforcement are treated concrete wall with 30 MB and dilating every 5 m with bituminized track d = 6mm.

After making the slope and wall approaches making footpath in a well compacted base. On a well pressed layer of compacted pad incorporates a layer of mixed concrete d = 5cm on which reinforcement is done with net Q188.

On the already installed reinforcement approach towards cementing the footpath MB 30 d = 12 cm dilation of 5m with (denote) bituminized track d = 3mm. As the final decoration on the footpath sets screed cement mortar and stone slabs with maximum thickness up to 6 cm.

After completion of works, the environment should be cleaned from the construction debris and settlement of the ground.

Measures of security and technical protection

In addition there will be included the necessary security measures and technical protection.

- To respect and enforce all laws and regulations in the operation of this type of work, and the regulations  written by the employer;

- To noticing all the necessary requirements and conditions that are part of this project, in terms of standards, regulations and legal provisions for performing this type of work;

- To take care for personal protection of the employees during the working actions  in accordance prescribed legal provisions and regulations;

- During transportation, all participants are obliged to use personal protective equipment;

- Constantly to carry out maintenance and cleaning of the riverbed;

All other prescribed legal provisions and regulations (Law on Safety and Health at Work Official Gazette no. 92/2007, as well as other laws and rulebooks) that are associated with this type of work and activities should be implemented.